[image: image3.emf]

Firmware Update Guide

For Solid State Drives (SSDs)
Document History
	Revision
	Release Date
	Purpose and/or Desc. Of Changes

	4.0
	July 23, 2009
	Fourth


CAUTION: Please backup data from your SSD before start update F/W. Super Talent is not held responsible for lost data. To ensure SSD data is safe, please backup all data on a separate drive before proceeding to update F/W on your SSD. 
This update tool can be only used to FTM56GL25H and FTD28GL25H.
· Limitation: This tool only works with SATA port configured to “Legacy/Compatility” mode or “Enhanced IDE” mode. “AHCI” mode is not supported.
F/W update via Booting Diskette
Firmware Files
1. Please download FWUPD.rar from the Super Talent website. 

2. Unzip FWUPD.rar to see the two files inside. The two files are:

· SATAISP.exe
· ISPB.LIB
Make a Booting Floppy Diskette
1. Windows 98 is recommended 

F/W Update Process

1. Copy the two unzipped files onto the floppy diskette

2. Insert the floppy diskette into the floppy drive

3. Connect the SSD with the SATA cable and SATA power cable

4. Turn on the computer. Check the BIOS settings to make sure that the floppy diskette boots up

5. Type “SATAISP” to run the F/W update tool

6. If the SSD is connected correctly, the message in Figure 1 will appear. 
[image: image1.jpg](SSD FAJ update tool ver B.0.9> (Jul B3 2009,17:03:25)
(System mode>

Flash mode |=KIHCCBBU1M

(Ch,Bank,CE ,DrvCur)=(8,8,2,3)
Part 1 : SATA
Part 2 : no code

Create original setting file : old.txt
FW updated count : Bx1

Current firmware version : 081289
New updated firmware version : 090331

This program will destroy all data in SSD, do umi want to continue (YAN)?





Figure 1: Update tool detects the SSD information
7. Press “Y” (if you already backup data or you don’t need)
8. The system will automatically update the F/W and format the SSD. Figure 2 is a screen capture of a successful F/W update.  The result will be saved to a file named “mo.txt”. 

9. After finished F/W update successfully, the new F/W version number should be 090331.

[image: image2.jpg]<SSD FZW update tool ver B.0.9> (Jul 83 2009,17 :03:25)
(System mode>

Flash mode |=KSHCGBBU1M

(Ch,Bank,CE ,DrvCur)=(8,8,2,3)
Part 1 : SATA

Part 2 : no code

Create original setting file : old.txt
FW updated count : Bx1

Current firmware version : 881289
New updated firmware version : 898331

This program will destroy all data in SSD, do you want to continue (Y/N)? ¥






Figure 2: Screen Shot of Successful F/W Update
Firmware Update via Bootable USB Drive

Firmware Files
1. Please download FWUPD.rar from the Super Talent website.

2. Unzip FWUPD.rar to see the two files inside. The two files are:

· SATAISP.exe
· ISPB.LIB 

Check CMOS Setting
1. Select the bootable USB drive as the first boot device

2. In order to update the F/W, the following BIOS settings must be set: 

· Integrated Peripherals

· PCI IDE BusMaster: (Enabled) Select

· On chip SATA Type: (Native IDE) Select

· Options: Native IDE / RAID / Legacy IDE (don’t select AHCI)
3. Save Changes
F/W Update Process
1. Copy the two unzipped files onto the bootable USB drive
2. Plug the USB drive into the USB connection into the computer

3. Connect the SSD with the SATA cable and SATA power cable

4. Turn on the computer and enter the correct CMOS settings (listed in above paragraph)

5. Boot to USB drive

6. Type “SATAISP” to run the F/W update tool  
7. If the SSD is connected correctly, the message in Figure 1 will appear

8. Press “Y” (if you already backup data or you don’t need)
9. The system will automatically update the F/W and format the SSD. Figure 2 is a screen capture of a successful F/W update. 
10. After finished F/W update successfully, the new F/W version number should be 090331.
Super Talent Technology, Inc.

2077 N. Capitol Ave., San Jose, CA 95132 ( Tel: (408)934-2560 ( Fax: (408)719-5020

www.supertalent.com

[image: image3.emf]